BRITISH COLUMBIA ELECTRICITY SUPPLY

BC HYDRO

| Generating Station (Hydroelectric) | Capacity (MW) | Location | Year Built | Average Energy Capability (GWh/year) |
| --- | --- | --- | --- | --- |
| Aberfeldie | 24 | Bull River, tributary of Kootenay River | 1922, rebuilt 1954, 2009 | 105 |
| Alouette | 9 | Alouette River, tributary of Pitt River, east of Vancouver | 1930 | 53 |
| Ash River | 27 | Ash River at outlet of Elsie Lake, central Van. Island; 6.5 km tunnel directs the water to power station on Great Central Lake | 1958 | 193 |
| Bridge River | 460 | Dam on Bridge River 40 km west of Lillooet; water diverted through tunnels to powerhouses on Seton Lake Reservoir | 1948-1960 | 2,666 |
| Buntzen/Coquitlam | 73 | Dam on Coquitlam River, tributary of Fraser River; 4 km tunnel takes water from Coquitlam Reservoir to Buntzen Lake which is dammed at its outlet; water from Buntzen Reservoir flows to the powerhouses on Indian Arm east of Vancouver | 1904 | 126 |
| Cheakamus | 158 | Outlet of Daisy Lake on Cheakamus River, tributary of Squamish River; 11 km tunnel directs water from Daisy Lake to powerhouse on upper Squamish River | 1957 | 638 |
| Clowhom | 33 | Head of Salmon Inlet, Sechelt peninsula | 1957 | 126 |
| Elko | 12 | Elk River, tributary of Kootenay River | 1924 | 79 |
| Falls River | 7 | Confluence of Big Falls Creek & Ecstall River, 25 km above Ecstall/Skeena confluence | 1930, upgrade 1983 | 49 |
| John Hart | 126 | Campbell River system, Vancouver Island | 1947 | 823 |
| Jordan River | 170 | Jordan River, sw coast Vancouver Island | rebuilt 1971 | 261 |
| Kootenay Canal | 580 | Midway between Nelson and Castlegar on Kootenay River. Water is diverted from Kootenay River to flow along canal to powerhouse, then returns to river | 1976 | 3,270 |
| La Joie | 25 | Dam on Bridge River near Gold Bridge at Downton Lake | 1956 | 172 |
| Ladore | 47 | Campbell River system, Vancouver Island | 1949-1957 | 280 |
| Mica | 1,750 | Dam & GS on Columbia River 135 km north of Revelstoke, creating Kinbasket Reservoir | 1973-1977 | 7,450 |
| Generating Station (Hydroelectric) | Capacity (MW) | Location | Year Built | Average Energy Capability (GWh/year) |
| Peace Canyon | 694 | Peace Canyon dam 23 km downstream of WAC Bennett dam, just upstream of Hudson’s Hope; impounds Dinosaur Lake, a narrow reservoir within Peace River canyon | 1980 | 3,375 |
| Puntledge | 24 | Puntledge River at outlet of Comox Lake | rebuilt 1956 | 159 |
| Revelstoke | 1,980 | Dam and GS built on Columbia River 5 km north of Revelstoke; formed Revelstoke Reservoir | 1984 | 8,400 |
| Ruskin | 105 | Stave River, 65 km east of Vancouver, tributary of Fraser River | 1930 | 374 |
| Seton | 48 | Water carried by canal from Seton Lake Res. to powerhouse on Fraser River | 1956 | 321 |
| Seven Mile | 804 | Pend d’Oreille River (tributary of Columbia River) upstream of Waneta dam | 1979-2003 | 3,523 |
| GM Shrum | 2,730 | WAC Bennett dam upstream of Hudson’s Hope, created Williston Lake | 1967 | 13,070 |
| Shuswap | 6 | Shuswap River near Lumby | 1929 | 40 |
| Spillimacheen | 4 | Spillimacheen River, tributary of Columbia River | 1955 | 23 |
| Stave Falls | 91 | Stave River, 65 km east of Vancouver, tributary of Fraser River | 1930, rebuilt 2000 | 362 |
| Strathcona | 65 | Campbell River system, Vancouver Island | 1958 | 246 |
| Wahleach | 64 | 25 km west of Hope, at outlet of Jones Lake Reservoir | 1952 | 282 |
| Walter Hardman | 8 | Cranberry Creek, w. side of northern end of Upper Arrow Lake (Columbia R.) | 1961 | 41 |
| Whatshan | 54 | Whatshan River, tributary of Lower Arrow Lake (Columbia River) | 1952 | 124 |
| TOTAL | 10,178 |  |  | 46,630 |
| Dependable capacity | ~9,707 |  |  |  |
| Firm energy capability |  |  |  | ~42,600 |

Stave River System

Includes 9 MW Alouette GS, 91 MW Stave Falls GS & 105 MW Ruskin GS.

System begins at Alouette Reservoir, formed by a dam on the Alouette River.

A 1 km tunnel carries water from Alouette Reservoir through Alouette power plant and into Stave Reservoir, a natural lake on Stave River raised by Stave Falls dam.

Water from Stave River flows through Stave Falls power plant into Hayward Reservoir, formed by Ruskin dam.

Water then flows through Ruskin power plant.

Bridge River System

Bridge River dammed by La Joie dam upstream of Hurley River at outlet of Downton Lake, forming Downton Reservoir.

Terzaghi dam 40 km upstream of Lillooet formed Carpenter Reservoir.  Water is diverted through two tunnels from Carpenter Reservoir to powerhouses at Shalalth on Seton Lake Reservoir.

Dam on Seton River at Seton Lake; water carried by canal to powerhouse on Fraser River.

Cayoosh Creek is diverted to Seton Lake Reservoir.

Campbell River System

Comprised of three power generating stations. Upper Campbell Lake Reservoir discharges through the 65 MW Strathcona GS into Lower Campbell Lake.  Lower Campbell Lake Reservoir is the headpond for the 47 MW Ladore GS.  Ladore discharges into John Hart Reservoir, the headpond for the 126 MW John Hart GS.  Upper Campbell Lake is the storage reservoir for all these installations.

Heber Creek and Crest Creek diversions provide additional flows into Upper Campbell Lake Reservoir.  Salmon River and Quinsam River diversions provide additional flows into Lower Campbell Lake Reservoir.

Peace River System

WAC Bennett dam created Williston Reservoir; GM Shrum GS; upstream of Hudson’s Hope; the Peace Canyon dam is 23 km downstream of WAC Bennett dam, just upstream of Hudson’s Hope; created Dinosaur Lake; Peace Canyon GS

The combined production at the two generating stations averages 16,336 GWh/year.

Columbia River System

1967	Duncan dam built above Kootenay Lake, creating Duncan Lake Reservoir in lower Duncan River, tributary of Kootenay River.  No generating station.

1969	Hugh Keenleyside dam built 8 km upstream of Castlegar on Columbia River, flooding Lower & Upper Arrow Lakes and creating one Arrow Reservoir.  No generating station.

1973	Mica dam and GS built on Columbia River 135 km north of Revelstoke, creating Kinbasket Reservoir and flooding Canoe River.  Produces an average of 8,381 GWh/year.

1976	Kootenay Canal GS built midway between Nelson and Castlegar on Kootenay River.  Water is diverted from Kootenay River at Corra Linn headpond to flow along man-made canal to powerhouse below South Slocan.

1979	Seven Mile dam and GS built on Pend d’Oreille River, tributary of Columbia River.

1984	Revelstoke dam and GS built on Columbia River 5 km north of Revelstoke; formed Revelstoke Reservoir.  Produces an average of 9,163 GWh/year.

| Generating Station (Thermal) | Capacity (MW) | Firm Energy Capability (GWh/year) | Location |
| --- | --- | --- | --- |
| Burrard (natural gas) | 913 (6 units) | 3,000 (3 units) | Vancouver |
| Prince Rupert (natural gas, diesel backup) | 46 | 180 | Prince Rupert |

Non-Integrated

| Generating Station | Capacity (MW) | Firm Energy Capability (GWh/year) | Location | Year Built |
| --- | --- | --- | --- | --- |
| Anaheim Lake (diesel) | 3.65 |  |  |  |
| Ah-Sin-Heek (diesel) & Clayton Falls (hydro) | 9.25 |  | Bella Coola |  |
| Bella Bella (diesel) | 2.7 |  |  |  |
| Bella Bella (hydro) | 6 |  |  |  |
| Atlin (diesel) | 2.65 |  |  |  |
| Dease Lake (diesel) | 3.98 |  |  |  |
| Dease Lake (hydro) | 3 | 5 | Hluey Lakes | 2002 |
| Eddontenajon (hydro) | 2.2 |  | Iskut |  |
| Fort Nelson (natural gas) | 47 | 155 |  |  |
| Ocean Falls (hydro) | 15 | 12 | Bella Bella |  |
| Pine Creek (hydro) | 2 | 4.6 | Atlin |  |
| Telegraph Creek (diesel) | 2.2 |  |  |  |
| Masset (diesel) | 11.37 |  |  |  |
| Sandspit (diesel) | 9.2 |  |  |  |
| Sandspit (hydro) | 6 | 20 |  |  |

FORTISBC

Four dams and generating stations on west arm of Kootenay Lake (Kootenay River system); from upstream to downstream:

- Corra Linn		built 1932		installed capacity 50 MW

- Upper Bonnington	built 1907, rebuilt 1940	installed capacity 59 MW

- Lower Bonnington	built 1898		installed capacity 41 MW

- South Slocan		built 1929		installed capacity 55 MW

205 MW; 1,500 GWh/year

Under the Canal Plant Agreement, the operations of these 4 stations, along with Waneta (Teck Cominco), Brilliant (Brilliant Power Corp.) and Kootenay Canal, are coordinated to produce the most power overall.  BC Hydro takes into its system all the power generated by all the plants, and the owners of the six non-BC Hydro plants receive Entitlements from BC Hydro.  FortisBC receives the amount of power its 4 generating stations would have produced without Kootenay Canal and the Columbia River Treaty.  Its entitlement in 2004 was 1,458 GWh.

COLUMBIA POWER CORPORATION / COLUMBIA BASIN TRUST

CPC is a crown corporation wholly owned and controlled by the Province of BC.  CPC undertakes power projects through joint ventures with Columbia Basin Trust.  CPC is manager of the JVs.  Each project is established as a separate corporation.  The JV power project companies, owned by CPC and CBT, are IPPs that are wholesalers of power.  The planning, design, financing, construction, operation and power sales functions involve private sector firms wholly or in part.  CPC follows a public-private-partnership (P3) model for the design, procurement and operation of the JV power projects.

The projects have ‘entitlement’ agreements with BC Hydro.  These entitlements provide the projects with predetermined energy and capacity quantities.  BC Hydro is compensated by keeping a small share of the annual energy produced.

Columbia Basin Trust is a partnership of local government, First Nations & the Province of BC.  It invests its 50% share of the JV net income to deliver benefits to the residents of the Columbia Basin.

Arrow Lakes GS	185 MW; 767 GWh/year		400 m downstream of Hugh Keenleyside dam, Columbia River

Completed 2002	Owned by Arrow Lakes Power Corp. (50/50 CPC and CBT Arrow Lakes Power Development Corp.)  Operated by Fortis Pacific.  12-year power sales agreement with BC Hydro for all the power, expiring 2015.

Brilliant dam & GS	145 MW; 980 GWh/year		Built 1944-1967; upgraded from 125 MW to 145 MW in 2000-2002	Downstream of the

other four dams and five GSs on west arm of Kootenay Lake, 8 km upstream of Castlegar	Owned by Brilliant Power Corp.

(50/50 ownership CPC & CBT Power).  Operated by FortisBC	Purchased from Cominco by the JV in 1996.  	60-year power

sales agreement with FortisBC for 125 MW / 859 GWh, expiring 2056.  FortisBC takes an additional 20 MW / 60 GWh/year from the upgrade; this can be recalled by CPC/CBT upon 90 days notice.  An additional 60 GWh from the upgrade is sold to Powerex.  Entitlement under Canal Plant Agreement 980 GWh.

Brilliant Expansion	120 MW; 500 GWh/year		New generating facility adjacent to Brilliant dam, commenced operation September 2007.     Owned by Brilliant Expansion Power Corp. (50/50 CPC and CBT Brilliant Expansion Power Corp.)	Part of the Canal Plant Agreement, whereby BEPC receives energy and capacity entitlements from BC Hydro in exchange for the actual project generation.	90% of its power has been sold to BC Hydro under two long-term agreements (108 MW / 425 GWh/y total); marketing of the remainder in domestic and US markets.

Waneta Expansion	up to 435 MW	Installation of an additional powerplant at the Waneta dam; construction may start fall 2009, commercial power production 2013		The rights to undertake the Expansion were purchased in 1994 by CPC and are now jointly owned with CBT.  Teck Cominco owns the Waneta dam.

TECK RESOURCES LTD.

Waneta dam & GS	475 MW	 Built 1954-1966	Upgrading of four generators completed January 2007, bringing capacity to 475 MW. Located on Pend d’Oreille River immediately above the confluence with Columbia River		Owned by Teck, operated by FortisBC	Under the Canal Plant Agreement, BC Hydro takes into its system all the generated power and provides entitlement to Teck.	Teck has the right to sell any surplus power; can sell to the US.		      Trail operations use 1,800 GWh/year.	In July 2009, BC Hydro applied to BCUC to purchase all Teck’s surplus power – 890 GWh/y firm, 167 MW dependable capacity.

CANAL PLANT AGREEMENT

Under the Canal Plant Agreement, the operations of the four FortisBC stations on the Kootenay River, Waneta (Teck Cominco), Brilliant (Brilliant Power Corp.) and Kootenay Canal are coordinated to produce the most power overall.  BC Hydro takes into its system all the power generated by the six non-BC Hydro stations, and the owners of these stations receive Entitlements from BC Hydro.  Brilliant Expansion also falls under the CPA.

RIO TINTO ALCAN

Kenney dam built on Nechako River southwest of Vanderhoof in the early 1950s; tunnel directs water from Nechako Reservoir to Kemano GS near the coast.		 Peak capacity 890 MW.

In January 2008, the 2007 EPA was approved, replacing the 1990 LTEPA.  Firm energy 162 aMW (1,419 GWh/y) 2007 – 2009, declining to 33 aMW (289 GWh/y) in 2017 - 2034.  Expected total power 225 aMW (1,971 GWh/y) 2007 – 2009, declining to 96 aMW (841 GWh/y) in 2017 – 2034.

COLUMBIA RIVER TREATY

Ratified 1964 between US and Canada.  To coordinate flood control and optimize electrical energy production in the Columbia River Basin in US and Canada.  Canada agreed to build three storage dams (Duncan, Hugh Keenleyside, Mica, completed 1973).  A fourth, Libby dam, was built in Montana, flooding into BC.  This allowed hydroelectric facilities in the US on the Columbia River to produce 2,400 MW of additional power capacity.  The Province of BC owns the Canadian entitlement; the province receives payments for the flood control benefits and 50% of the additional power benefits (the downstream benefits) generated at US projects.  The Province received $64.4 million; and the first 30-years of the downstream benefits was sold to US utilities for $254 million (US).  This sales agreement began to expire in 1998.  The province is now receiving the Canadian entitlement for the remaining 30 years of the CRT.  Although there is no official ‘expiry date’, there are provisions for renewal, termination or re-negotiation after 60 years (2024), provided the parties give 10 years notice (2014).  Canada is obligated to continue to provide flood control as long as the three CRT dams are in operation.

ISLAND COGENERATION PROJECT

Owned by Calpine Canada Power (30%)	Operational April 2002		Located at Elk Falls, Vancouver Island		Natural gas-fired Supplies steam to Norske Canada mill and electricity to BC Hydro	Contractual with BC Hydro 265 MW / 1,900 GWh/year through 2022

OTHER IPPs

The power is non-dispatchable, i.e., BC Hydro has no control over timing of energy supply.

| Generating Station | Capacity (MW) | Energy (GWh/yr) | Location | Year Completed |
| --- | --- | --- | --- | --- |
| Akolkolex Hydro | 8 | 50 | Revelstoke | 1995 |
| Boston Bar Hydro | 6 | 38 | Scuzzy Creek | --- |
| Brandywine Creek Hydro | 8 | 34 | Tributary of Cheakamus River | 2003 |
| Brown Lake Hydro | 7 | 57 | Prince Rupert | 1996 |
| China Creek Hydro | 6 | 25 | Port Alberni | 2005/06 |
| Coats | <0.5 | 1 | Gabriola Island | 1980s |
| Doran-Taylor Hydro | 6 | 23 | Port Alberni | 1996 |
| Eagle Lake Hydro | 0.2 | 1 | West Vancouver | 2003 |
| East Twin Hydro | 2 | 6 | McBride | 1990s |
| Furry Creek Hydro | 10 | 40 | Lions Bay | 2004 |
| Hauer Creek Hydro (aka Tete) | 2 | 12 | Valemount | 2007 |
| Hystad Creek Hydro | 6 | 20 | Valemount | 2002 |
| Lower Mamquam Hydro | 58 | 250 | Squamish | 1990s |
| McDonald Ranch | <0.5 | <0.5 | Grassmere | 1990s |
| Marion 3 Creek Hydro | 5 | 18 | Port Alberni | 2005 |
| McNair Creek Hydro | 10 | 38 | Port Mellon | 2004 |
| Mears Creek Hydro | 4 | 20 | Tributary of Gold River | 2004 |
| Miller Creek Hydro | 30 | 118 | Pemberton | 2003 |
| Morehead Creek Hydro | 0.1 | <0.5 | Morehead Creek, Williams Lake | 1994 |
| Pingston Creek Hydro | 45 | 193 | Revelstoke | 2003 |
| Raging River 1 Hydro (storage) | 2 | 13 | Port Alice | 2002 |
| Robson Valley Power | 4 | 26 | McBride | 1990s |
| Rutherford Creek Hydro | 50 | 172 | Pemberton | 2004 |
| Seaton Creek Hydro | <0.5 | 1 | New Denver | 1990s |
| Sechelt Creek Hydro | 12 | 68 | Sechelt | 1997 |
| Soo River Hydro | 12 | 65 | Whistler | 1994 |
| South Cranberry Creek Hydro | 8 | 33 | Revelstoke | 2008 |
| South Sutton Creek Hydro | 5 | 26 | Port Alberni | 2005 |
| Upper Mamquam Hydro | 25 | 108 | Squamish | 2005 |
| Walden Hydro | 18 | 54 | Cayoosh Creek | 1990s |

| 150 Mile House ERG (gas-fired) | 6 | 34 | 150 Mile House | 2009 |
| --- | --- | --- | --- | --- |
| Armstrong Woodwaste Cogen | 20 | 64 | Armstrong | 2003 |
| Hartland Landfill | 2 | 14 | Saanich | 2004 |
| McMahon Cogeneration | 105 | 840 | Taylor | 1990s |
| Purcell Power Biomass | 43 | 85 | Skookumchuk | 1990s |
| Savona ERG (gas-fired) | 6 | 41 | Savona | 2009 |
| SEEGEN Municipal Solid Waste | 22 | 131 | Burnaby | 2003 |
| Vancouver Landfill Gas | 8 | 55 | Delta | 2005 |
| Williams Lake Biomass | 66 | 545 | Williams Lake | 1993 |

RESOURCE SMART

Investment at existing hydroelectric facilities.

Projects:		GM Shrum 6 to 8						90 MW

Revelstoke 5		scheduled completion fall 2010		120 GWh/year/ 480 MW

Mica 5			estimated completion fall 2013		500 MW

Mica 6								500 MW

Cheakamus 2							46 GWh/year

John Hart